## 08 11 13 - Hollow Metal Doors and Frames

### Steel Doors:

- Exterior:  Full flush doors (ANSI A250.8-1998 (SDI-100)), 1-3/4 inches thick,  galvannealed steel (ASTM A653/A653M-07), 14 gage closer reinforcement top and bottom with snap-in top caps.

- Interior:  Full flush doors (ANSI A250.8-1998 (SDI-100)), 1-3/4 inches thick, 14 gage closer reinforcement top and bottom.

- Hardware preparation and reinforcement:  Prepare for commercial hardware (ANSI A250.6-1997) and mortised locks (ANSI A115.1), no face holes unless coordinated with hardware schedule.  Factory reinforce for surface applied hardware – 14 gage for closer.  Locations per ANSI/DHI A115.

- Preferred Manufacturer and Model:  Steelcraft L16 Series

### Steel Door Frames:

- Exterior:  16 gage welded door frames; A60 hot dipped galvannealed steel.

- Flush Frames:  F-Series Frames, 16 gage. http://www.steelcraft.com/downloads/pdfs/steelcraft_spec_l1.pdf

- Interior:  16 gage knock-down door frames.

  - Drywall Frames: DW-Series (using Baseboards) and K-Series (Not using Baseboards), 16 gage.

      - http://www.steelcraft.com/pc_frames_drywall.asp

      - http://www.steelcraft.com/downloads/pdfs/steelcraft_spec_dw1.pdf

- Reinforcement:  14 gage for closer, 4-7/8 inches ASA strike, and 4-1/2 inches hinges.

- Preferred Manufacturer:  Steelcraft.

Timely door frames are not preferred.