- Section 33 42 11 - Stormwater Gravity Piping

  - PART 1 GENERAL

    - REFERENCE STANDARDS

      - ASTM D3034 - Standard Specification for Type PSM Poly Vinyl Chloride (PVC) Sewer Pipe and Fittings 2016.

    - SUBMITTALS

      - See Section 01 30 00 - Administrative Requirements, for submittal procedures.

      - Manufacturer's Certificate: Certify that products meet or exceed specified requirements.

  - PART 2 PRODUCTS

    - REGULATORY REQUIREMENTS

      - Conform to applicable code for materials and installation of the Work of this section.

    - SEWER PIPE MATERIALS

      - Plastic Pipe: ASTM D3034, Type PSM, Poly Vinyl Chloride (PVC) material; inside nominal diameter of [____] inches ([____] mm), bell and spigot style solvent sealed joint end.

    - PIPE ACCESSORIES

      - Pipe Joints: Mechanical clamp ring type, stainless steel expanding and contracting sleeve, neoprene ribbed gasket for positive seal.

      - Fittings: Same material as pipe molded or formed to suit pipe size and end design, in required tee, bends, elbows, cleanouts, reducers, traps and other configurations required.

    - CATCH BASIN, TRENCH DRAIN, CLEANOUT, AND AREA DRAIN COMPONENTS

      - Lids and Drain Covers: Cast iron, hinged to cast iron frame.

        - Catch Basin:

          - Lid Design: Linear grill.

          - Nominal Lid and Frame Size: 26 inches (660 mm) diameter.

        - Cleanout:

          - Lid Design: Linear grill.

          - Nominal Lid and Frame Size: 26 inches (660 mm) diameter.

        - Area Drain:

          - Lid Design: Linear grill.

      - Trench Drain System: Trench drain system assembled from factory fabricated, polymer concrete castings in standard lengths and variable depths, with integral joint flanges and integral grating support rails; includes joint gaskets and grating.

        - Trench Width: 4 inches (102 mm).

        - Trench Section Length: 39 inches (1 m) and 19-1/2 inches (500 mm).

        - Grating Support Rail: Stainless steel.

  - END OF SECTION  33 42 11