## 09 97 23 - Concrete and Masonry Coatings

### Exterior Concrete Masonry Units:

Provide the following finish systems over exterior concrete masonry units:

Option 1:

1st Coat: Heavy Duty Int/Ext Acrylic Block Filler B42W46

2nd Coat: Super-Paint Exterior Acrylic Satin Enamel A89W1151 Series

3rd Coat: Super-Paint Exterior Acrylic Satin Enamel A89W1151 Series

Option 2:

1st Coat: Heavy Duty Int/Ext Acrylic Block Filler B42W46

2nd Coat: DE Evershield Exterior Eg-Shel Paint EVSH 30-2

3rd Coat: DE Evershield Exterior Eg-Shel Paint EVSH 30-2

### Interior Concrete Masonry Units:

Provide the following finish system over interior concrete masonry block units:

Option 1:

1st Coat: PrepRite Int/Ext Acrylic Block Filler B25W25

2nd Coat: Super-Paint Interior Acrylic Satin Enamel A87W1151 Series

3rd Coat: Super-Paint Interior Acrylic Satin Enamel A87W1151 Series

Option 2:

1st Coat: PrepRite Int/Ext Acrylic Block Filler B25W25

2nd Coat: DE Everest Interior Eg-Shel Paint EVER 30

3rd Coat: DE Everest Interior Eg-Shel Paint EVER 30